## 11 PAIN POINTS KRITIS

| # | Pain Points | Impact |
| --- | --- | --- |
| 1 | Material shortage tiba-tiba | Production stop, overtime, emergency purchasing |
| 2 | Tidak tahu WIP real-time | Planning impossible, bottleneck detection delayed |
| 3 | Quality defect traceable manual (Excel terpisah) | Tidak terintegrasi, IKEA audit risk, sulit root cause analysis |
| 4 | Mix Label risk (Finishing-Packing) | IKEA REJECT! Salah lihat datestamp/destination, no validation |
| 5 | Dual trigger system manual | Admin overwhelmed, packaging error frequent |
| 6 | Excel-based planning | Error prone, single point of failure |
| 7 | Target produksi flat | Shortage frequent (tidak hitung defect rate) |
| 8 | Manual material tracking | Admin time wasted 2-3 jam/hari |
| 9 | Stock opname 1 hari penuh | Production stop untuk counting |
| 10 | Rework tidak ter-record | Cost tidak tahu, IKEA audit concern |
| 11 | Multi-unit conversion manual (Purchasing & Production) + Pallet calculation | UOM error: Purchasing terima ROLL consume METER, Thread KG consume GRAM. Pallet IKEA rules complex (24 karton genap per pallet). Manual Excel formula prone error! |

CRITICAL: Pain point #4 (Mix Label) adalah high-risk untuk IKEA compliance! Operator sering salah lihat Week/Destination di Finishing & Packing, menyebabkan mix label saat dispatch. Tidak ada pengecekan kembali setelah packing selesai → IKEA reject baru ketahuan!

## 7 UNIQUE REQUIREMENTS

### Dual Purchase Order Trigger System

Purchasing create PO Fabric (day 0) → Validate → Unlock Cutting & Embroidery ONLY

Purchasing create PO Label (days +3) → Validate → Unlock ALL dept + Auto-inherit Week/Destination

Standard ERP: 1 MO = 1 trigger. Kami butuh: 1 MO = 2 PO triggers dengan state management (PARTIAL → FULL RELEASED)

### Flexible Production Target per Department

Setiap dept produksi LEBIH dari customer order (cover defect)

Auto-calculate target based on historical defect rate per dept

Reverse calculation: Packing need 1000 → Finishing 1020 → Sewing 1040

### 2-Stage Finishing Internal Conversion

Stage 1: Stuffing (Skin + Filling → Stuffed Body)

Stage 2: Closing (Stuffed Body + Tag → Finished Doll)

Track stock intermediate product TANPA transfer document

### Multi-Unit Conversion & Pallet Calculation (IKEA Rules)

Fabric: Beli ROLL, pakai PCS (1 ROLL = 50 meter, tapi vendor berbeda!)

Filling: Beli KG, pakai GRAM (conversion automatic)

Pallet: Calculate dari PCS → Karton → Pallet (IKEA rules: karton per pallet HARUS GENAP!)

Example: Order 1523 pcs → System calculate: Berapa pallet? Sisa karton genap?

Auto-convert + validate untuk prevent error

PO dibuat berdasarkan pallet calculation (bukan flat quantity)

### Real-Time WIP Tracking System

Dashboard real-time per dept, per MO, per SKU

Partial transfer support (tidak tunggu full batch)

Bottleneck alert automatic

### Quality Control Loop (Rework/Repair)

QC recording: Pass / Rework / Reject

Rework workflow dengan traceability

Defect tracking untuk IKEA compliance

### Department-Level Warehouse & Stock Opname

Stock opname per dept (tidak stop ALL production)

Rolling schedule: Today Cutting, Tomorrow Embroidery

Production continue di dept lain